| AES-2362-C4 | Intel Tiger Lake i5-1145GRE/i3-1115GRE CPU Fanless Industrial Computers, 2HDMI, 2LAN 4USB3.1/4USB2, 4COM, M.2, SIM,DC-IN 12-24V |
| --- | --- |

|  |  | Features - Intel 11th Tiger Lake-UP3 i5-1145GRE, i3-1115GRE CPU - Supports DDR4 2400MHz, 2xSO-DIMM, up to 64GB system memory - HDMI 2.0 port, max resolution up to 4096x2160 @60Hz - High-speed connectivity by dual 2.5GbE Intel LAN and 4 USB3.1 - 4x COM, COM1/2 by RS-485/422/232, COM3/4 by RS-232 - Onboard M.2 B/E/M-Key, Nano SIM card slot, TPM 2.0 - Wide DC 12-24V Input by 2-Pin phoenix connector - Wide Temperature component design -40C ~ 70C - ATM, Kiosk, self-service, machine automation, IoT gateway platform |
| --- | --- | --- |

# Specifications

| AES-2362 Series Key Features |  | AES-2362H-C4 | AES-2362E-C4 | AES-2362L-C4 |  | AES-2362C-C4 |
| --- | --- | --- | --- | --- | --- | --- |
| Processor System | CPU Max. Speed | Up 4.4GHz/1.8GHz | Up 4.1GHz/1.5GHz | Up 3.9GHz/2.2GHz |  | 1.8GHz |
|  | L2 Cache, Core | 12MB, 4-Core/8T | 8MB, 4-Core/8T | 6MB, 2-Core/4T |  | 4MB, 2-Core/2T |
|  | CPU/ TDP 12W/15W | i7-1185GRE | i5-1145GRE | i3-1115GRE |  | Celeron 6305UE |
|  | BIOS | 256Mb SPI MX25L25673GM21-08G |  |  |  |  |
|  | Graphics Processor | Intel UHD Graphics 620 |  |  | Intel UHD Graphics 610 |  |
| Expansion Slot | M.2 B-Key | 1 x M.2 B-Key 2242/3042/2280 (SATA 3.0/USB 3.0/USB 2.0 with Nano SIM) Slot |  |  |  |  |
|  | M.2 E-Key | 1 x M.2 E-Key 2230(PCI-E x1/USB2.0/CNVi) for Wireless |  |  |  |  |
|  | M.2 M-Key | 1 x M.2 M-Key 2242/2280 ( PCI-E x4 ), NVMe support (NVMe x4 ) |  |  |  |  |
| Memory | Specification (SO-DIMM) | 2 x DDR4 2400MHz, 2x SO-DIMM, 32GB per slot |  |  |  |  |
|  | System Max. Capacity | 64GB |  |  |  |  |
| Graphics | DP(from Thunderbolt) | 1 x DP++ by Thunderbolt 3 USB-C(Alt mode, resolution up to 4096x2340 @ 60Hz) |  |  |  |  |
|  | HDMI | 1 x HDMI 2.0, 1 x HDMI 1.4b (max. resolution up to4096 x 2160@60Hz) |  |  |  |  |
| Ethernet | LAN Gigabit Controller | 2 x LAN (Intel I225-IT 2.5G, TSN, industrial grade) |  |  |  |  |
| Storage | M.2 B-Key | 1 x M.2 B-Key 2242/2280 ( SATA 3.0/USB3/USB2) with Nano SIM Holder |  |  |  |  |
|  | M.2 M-Key | 1 x M.2 M-Key 2242/2280(NVMe x4) |  |  |  |  |
|  | 2.5” SATA Driver | N/A |  |  |  |  |
| System Rear I/O | HDMI | 2 x HDMI |  |  |  |  |
|  | Ethernet | 2 x RJ45 |  |  |  |  |
|  | USB Type A | 3 x USB 3.1(Gen 2), 2 x USB 2.0 |  |  |  |  |
|  | Audio | 1 x Audio port (Line-out) |  |  |  |  |
|  | System Power Input | 1 x 2-pin phoenix connector 12-24Vdc Input |  |  |  |  |
|  | Antenna | 2 x Antenna holes (options) |  |  |  |  |
| System Front Bezel | Power-On Button | 1x Moment button switch to turn-on system |  |  |  |  |
|  | System Power LED | 1x LED for system power status |  |  |  |  |
|  | HDD LED | 1x LED for SSD data access status |  |  |  |  |
|  | COM1-2 Port | 2 x DB-9 ( 2 x RS-485/422/232, Pin9 +5/+12V selectable,) |  |  |  |  |
|  | COM3-4 Ports | 2 x DB-9 ( 2 x RS-232) |  |  |  |  |
|  | USB Type A | 2 x USB 2.0 |  |  |  |  |
|  | Option GPIO 8-bit | 1 x DB-9 or 1x GPIO Board by request on front bezel, 4 x GPI + 4 x GPO and 5V/GND |  |  |  |  |
|  | Customized LVDS Port | Customized LVDS solution for open frame LCD display by request |  |  |  |  |
| Operation System | Win 10 | Win 10 IoT 64-bit, Ent 2019 LTSC |  |  |  |  |
| Watchdog Timer | Output and Interval | System reset, programmable 1 ~ 255 sec/min; 1 sec. or 1min. /step |  |  |  |  |
| Power Requirement | DC Input | 12-24Vdc input (option DC12V 60W adapter) |  |  |  |  |
| Environment | Operating Temperature | 0°C ~50°C(32~122°F) |  | TBD |  |  |
|  | Air Flow condition | 50°C - 60°C (122~140°F) w/0.7m/s air flow |  | TBD |  |  |
|  | Storage Temperature | -40 ~ 85°C ( -40 ~ 185°F ) |  |  |  |  |
| Physical Size | Dimensions | 225mm(W) x 140mm(D) x 45mm(H) |  |  |  |  |
| Packing Size | Dimensions | 300mm(W) x 270mm(D) x 130mm(H) |  |  |  |  |
| Weight | N.W / G.W. | Single packed: 2.5Kg / 3Kg |  |  |  |  |
| Certifications | EMC | N/A |  |  |  |  |

# Dimensions Outlook

## Ordering Information

| Part Number | CPU | HDMI | 2.5 G | USB 3.1 | USB 2.0 | COM | DDR4 RAM | M.2 B-Key | M.2 M-Key | 60W PSU |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| AES-2362H-C4-M0-M00P0 | i7-1185G7E | 2 | 2 | 4 | 4 | 4 | Option | Option | Option | Option |
| AES-2362E-C4-M0-M00P0 | i5-1145GRE | 2 | 2 | 4 | 4 | 4 | Option | Option | Option | Option |
| AES-2362L-C4-M0-M00P0 | i3-1115GRE | 2 | 2 | 4 | 4 | 4 | 4GB | Option | Option | Option |
| AES-2362C-C4-M0-M00P0 | C-6305UE | 2 | 2 | 4 | 4 | 4 | 4GB | Option | Option | Option |
| AES-2362H-C4-M8-R12DP0 | i7-1185GRE | 2 | 2 | 4 | 4 | 4 | 8GB | Option | 128G TLC/80 x4 | Option |
| AES-2362E-C4-M8-N12DP0 | i5-1145GRE | 2 | 2 | 4 | 4 | 4 | 8GB | 128G TLC/80 SA | Option | Option |
| AES-2362L-C4-M8-N64DP0 | i3-1115GRE | 2 | 2 | 4 | 4 | 4 | 8GB | 64G TLC/80 SA | Option | Option |
| AES-2362C-C4-M0-M64DP0 | C-6305UE | 2 | 2 | 4 | 4 | 4 | 4GB | 64G TLC/42 SA | Option | Option |

## Optional Items

| Part Number | Description |
| --- | --- |
| SODIMM-DDR4- M4 / M8 | 204-Pin SODIMM DDR4-2133, 4GB / 8GB |
| M2-M64D(2242)/N12D(2280) | M.2 SATA3 2242 64GB 3D-TLC/ M.2 2280 128GB 3D-TLC (B-KEY ) |
| M2-M64L(2242)/N12L(2280) | M.2 SATA3 2242 64GB MLC/ M.2 2280 128GB MLC (B-KEY) |
| M2-O64L(2242)/P12L(2280) | M.2 NVMe Gen3x2, 2242 64GB MLC/ M.2 NVMe Gen3x2, 2280 128GB MLC (M-KEY ) |
| M2-Q12D(2242)/R12D(2280) | M.2 NVMe Gen3x4, 2242 128GB 3D-TLC / M.2 NVMe Gen3x4 2280 128G 3D-TLC(M-KEY ) |
| AC-9260 | M.2 2230 (E-KEY) Wi-Fi Module, Intel AC9260 802.11abgn + Bluetooth 5.0, dual band 2T2R |
| DHAN3-60W-2P | AC-DC 60W Adapter, 90V-264Vac input by C14 socket, DC 12V@5A output with 2-pin phoneix connector |
| AES-WM-BK | AES Wall Mount Kits with Screws |